| Temperature Switch TWR |  |  |
| --- | --- | --- |
|  | Download PDF datasheet | Switch points: +30 ... +120 (in 5 or 10 °C steps) fixed Connection:G ¾ male, brass or st. st. Electrical connection: plug DIN 43 650 Switching Accuracy: 30..+90 °C ± 5 °C 100-120 °C ± 7 °C Hysteris: max. 20 °C |
| Thermal Reed Switches TRS |  |  |
|  | Download PDF datasheet | Switch points: +10 ... +120 (in 5 bzw. 10 °C Schritten) fixed Connection:G ¼...G 1 male, brass or st. st. Electrical connection: cable Switching Accuracy: ± 3 °C Hysteresis: < 7 °C |
| Temperature Switch-Digital TDD-1,-3,-5,-7 |  |  |
|  | Download PDF datasheet | Switch points: -20 ... +125 °C, adjustable Connection:G ½, G¾ male, ½, ¾ NPT st. st. Indication: 3-digit LED Electrical connection: plug M12x1 or cable Supply: 24 VDC Contact: 1 or 2 semiconductors PNP/NPN N/O or N/C adjustable Switching Accuracy: ±0, 5 °C (-10...+85 °C) |
| Temperature Switch-Digital TDD-D6 |  |  |
|  | Download PDF datasheet | Switch points: -20 ... +125 °C, adjustable Connection:G 1/2, G3/4 male, 1/2, ¾ NPT st. st. Indication: 3-digit LED Electrical connection: plug M12x1 or cable Supply: 24 VDC Contact: 1 or 2 semiconductors PNP/NPN N/O or N/C adjustable Switching Accuracy: -10...+85 °C, ±0, 5 °C |
| Glass Thermometer for Machines TGL |  |  |
|  | Download PDF datasheet | Measuring range: -60 ... +40 up to 0 ... 200 °C Connection: G ½ or ½ NPT, brass Housing Material: aluminum Housing length: 110, 150 or 200 mm Accuracy class: 1,0 |
| Capillary Thermometer according to DIN 16205 TNF |  |  |
|  | Download PDF datasheet | Measuring range: -20 ... +40 up to 0 ... 600 °C Connection: back or bottom G ½... G1, ½ NPT...1 NPT, TriClamp, and so on Housing Material: aluminum, st. st., plastic, steel Diameter of housing: 63, 80, 100,160 or 250 mm Accuracy class: 1,0 or 1,6 Option: contacts, gliding pointer, oil filling, and so on |
| Safety Thermometers with Contact TNS, TNF |  |  |
|  | Download PDF datasheet | Measuring range: -20 ... +40 up to 0 ... 600 °C Connection: back or bottom G ½... G1, ½ NPT...1 NPT, TriClamp, and so on Housing Material: aluminum, st. st., plastic, steel Diameter of housing: 63, 80, 100,160 or 250 mm Accuracy class: 1,0 or 1,6 Option: contacts, gliding pointer, oil filling |
| Stem Thermometer for Diesel Engines TND |  |  |
|  | Download PDF datasheet | Measuring range: 0 ... +600 up to 0 ... 800 °C Connection: bottom, G ½, G ¾ Housing Material: steel or st. st. with silicone filling Diameter of housing: 63, 80, 100 mm Accuracy class: 1,6 (63, 80 mm), 1,0 (100 mm) Option: damping spring |
| Temperature sensor TDA |  |  |
|  | Download PDF datasheet | Measuring range: -50 ... +125 °C Connection:G 1/2, G3/4 male, 1/2, ¾ NPT st. st. Material: st.st. Indication: 3-digit LED Electrical connection: plug M12x1 or cable Supply: 24 VDC analogue output 0(4)-20 mA, programmable, Semiconductor PNP/NPN, digital display Accuracy: ±0, 5 °C (for range -10...+85 °C) |
| Infrared Hand-Held Thermometer TIR-HN |  |  |
|  | Download PDF datasheet | Non- contact temperature measurement Measuring range: -20 ... +500 up to -32 ... 900 °C Measuring Accuracy: ± 1 % up to ±2 % of m. value Option: analogue output, RS 232, data memory, Laser pointer, statistical functions |
| Precision Hand-Held Thermometer HND-T120 |  |  |
|  | Download PDF datasheet | Measuring range: -50 ... +1150 °C Probe: Thermo couples K Accuracy: 1 % Supply: 9 V battery |
| Precision Hand-Held Thermometer HND-T115,-T215 |  |  |
|  | Download PDF datasheet | Measuring range: -220 ... +1370 °C, -50..+1750°C Probe: Thermo couples Accuracy: ± 0,03 % of m. value Supply: 9 V battery Holding Function, min/max. -memory, °C/°F data logger, RS 232 |
| Room Thermometers TWL-ST |  |  |
|  |  | Measuring range: -20...+60 °C Connection: wall mounting Sensor: Pt 100, class A or B, 4-20 mA |

Search product types

| Pipe Temperature Sensor for Reliable Dead-Zone-Free TWP |  |  |
| --- | --- | --- |
|  | Download PDF datasheet | Measuring range:-20 ... +200 °C Connection: Hygenic connection, Triclamp Pipe: st. st. DN 15..DN 80 Sensor: Pt100 Head B or field housing |
| Thermocouples (according to DIN) TTD |  |  |
|  | Download PDF datasheet | Predection tube Form AM, AK, B, BM, MA Measuring range: -200.. +600 up to -200... 1150 (1200) °C Connection: G ½, sliding flange Material of predective tubes: steel, st. st., ceramics Sensor: FeCu-Ni, NiCr-Ni, class 2 Option: head mounted transmitter 4-20 mA |